Projekt Munka tesztfázis

Topológia

Weboldalunk:

https://lajkak2006.github.io/Taco-s-Kft./tacos.html

IP tábla

| Eszköz | Interface | IP | IPv6 | Subnet | Gateway |
| --- | --- | --- | --- | --- | --- |
| Center | Gig/0/0 Se0/1/0 Se0/1/1 | 192.168.7.1 20.0.0.1 30.0.0.1 | ------------------------------- ------------------------------------------------------------- | /27 /30 /30 |  |
| Delivery R1 | Gig0/0.40 Se0/1/0 | 192.168.5.65 20.0.0.2 | 2001:ACAD:DB8:5::2/64 ------------------------------- | /27 /30 |  |
| Delivery R2 | Gig0/0.50 Se0/1/0 | 192.168.5.97 30.0.0.2 | 2001:ACAD:DB8:5::1/64 ------------------------------- | /28 /30 |  |
| ConR | Gig0/0 Gig0/1.60 | 192.168.6.2 192.168.6.17 | ------------------------------- ------------------------------- | /28 /28 | --------------------- --------------------- |
| Boss Wi-Fi | Internet LAN | 192.168.7.2 192.168.7.33 | ------------------------------- | /29 /29 |  |
| DeliverySW1 | VLAN 40 | ------------------- ----- | -------------- | --------- |  |
| DeliverySW2 | VLAN 50 | ------------------- ----- | -------------- | --------- | --------------------- -- |
| DeliveryPC1 | NIC | DHCP | SLAAC | /27 | 192.168.5.65 |
| DeliveryPC2 | NIC | DHCP | SLAAC | /27 | 192.168.5.65 |
| DeliveryPC3 | NIC | DCHP | SLAAC | /28 | 192.168.5.97 |
| DeliveryPC4 | NIC | DHCP | SLAAC | /28 | 192.168.5.97 |
| DeliveryServer | NIC | DHCP | SLAAC | /27 | 192.168.5.85 |
| Laptop | Wireless | DHCP | -------------- | /29 | 192.168.7.1 |
| Smartphone | Wireless | DHCP | -------------- | /29 | 192.168.7.1 |
| ConPC | NIC | 192.168.6.20 | Stateful | /28 | 192.168.6.17 |
| ConServer | NIC | 192.168.6.25 | 2001:ACAD:DB8:6::1/64 | /28 | 192.168.6.17 |

Vlan tábla

| Eszköz neve | Azonosító | Név | Portok | Hálózat |
| --- | --- | --- | --- | --- |
| DeliverySW1 | 40 99 | Delivery1 Trash | Fa0/5,Fa0/10,Fa0/3 Fa0/6-9,fa0/11- 24,Gig0/2 | 192.168.5.64/27 |
| DeliverySW2 | 50 99 | Delivery2 Trash | Fa0/5,Fa0/10 | 192.168.5.96/28 |
|  |  |  | Fa0/3-4,Fa0/6- 9,Fa0/11-24,Gig0/2 |  |
| ConSW | 60 99 | ConVLAN Trash | Fa0/1, Fa0/2 Fa0/3-24,Gig0/2 | 192.168.6.16/28 |

EtherChannel

| Protokoll | Port |
| --- | --- |
| LACP | Fa0/1, Fa0/2 |
| LACP | Fa0/1, Fa0/2 |

Alapbeállítások:

Center:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class DeliveryR1:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class DeliveryR2:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class DeliverySW1:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class DliverySW2:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class ConR:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class ConSW:

- Password: cisco

- Minden jelszó titkosítva

- Konzol: console

- Telnet: class

Linux szerver:

Your name: taco_manager

Servers name: tacos_kft

Username: taco_loco

Password:tastytaco Groups: taco_managers Users:

- taco_lajka

- taco_varadi

- taco_enyedi

OSPF:

Center:

- Router ID: 10.10.10.10

- Folyamatazonosító: 10

- Hálózatok megadva: o 20.0.0.0/30 o 30.0.0.0/30 o 192.168.7.0/28 DeliveryR1:

- Router ID: 1.1.1.1

- Folyamatazonosító: 1 ➢ Hálózatok megadva: o 20.0.0.0/30 o 192.168.5.0/26 o 192.168.5.64/27 DeliveryR2:

- Routerd ID: 2.2.2.2

- Folyamatazonosító: 2 ➢ Hálózatok megadva: o 30.0.0.0/30 o 192.168.5.0/26 o 192.168.5.96/27 o 192.168.6.0/28 ConR:

- Router ID: 3.3.3.3

- Folyamatazonosító: 3 ➢ Hálózatok megadva: o 192.168.6.0/28 o 192.168.6.16/28

SSH:

DeliveryR1:

domain-name: taco.hu hostname: DeliveryR1 user admin secret admin crypto key: 1024 version 2 line vty 0 15: login local transport input ssh

DeliveryR2:

domain-name: taco.hu hostname: DeliveryR2 user admin secret admin crypto key: 1024 version 2

line vty 0 15: login local transport input ssh

ConR:

domain-name: taco.hu hostname: ConR user admin secret admin crypto key: 1024 version 2

line vty 0 15: login local transport input ssh

Center:

domain-name: taco.hu hostname: Center

user admin secret admin crypto key: 1024 version 2 line vty 0 15: login local transport input ssh

NAT:

HSRP:

DeliveryR1

DeliveryR2

Port biztonság:

DeliverySW1 1/2

DeliverySW1 2/2

DeliverySW2 1/2

DeliverySW2 2/2

ConsSW

Wi-Fi beállítások:

ACL:

A 192.168.7.0-s hálózat csak weboldalon érje el a 192.168.6.21-es Webserver-t

Etherchannel:

DeliverySW1:

DeliverySW2:

Configok

Center

!

version 15.1

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

hostname Center

!

!

!

enable password 7 0822455D0A16

!

!

!

!

!

!

no ip cef no ipv6 cef

!

!

!

username DeliveryR1 password 7 082048430017 username DeliveryR2 password 7 082048430017

!

!

license udi pid CISCO1941/K9 sn FTX1524U846-

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0  ip address 192.168.7.1 255.255.255.224  duplex auto  speed auto

interface GigabitEthernet0/1  no ip address  duplex auto  speed auto

!

interface Serial0/1/0  ip address 20.0.0.1 255.255.255.252  encapsulation ppp  ppp authentication chap  no keepalive  ip access-group 101 in

!

interface Serial0/1/1  ip address 30.0.0.1 255.255.255.252  encapsulation ppp  ppp authentication chap  ip access-group 101 in

!

interface Vlan1  no ip address  shutdown router ospf 10  router-id 10.10.10.10  log-adjacency-changes

passive-interface GigabitEthernet0/0  network 20.0.0.0 0.0.0.3 area 0  network 30.0.0.0 0.0.0.3 area 0  network 192.168.7.0 0.0.0.31 area 0

!

ip classless

ip route 0.0.0.0 0.0.0.0 GigabitEthernet0/1

!

ip flow-export version 9

!

!

access-list 101 deny ip any 0.0.0.0 255.255.255.240

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

line con 0  password 7 082243401A160912

login

!

line aux 0

!

line vty 0 4

password 7 0822404F1A0A

login line vty 5 15  password 7 0822404F1A0A

login

!

!

!

end

DeliveryR1:

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec service password-encryption

!

hostname DeliveryR1

!

!

!

enable password 7 0822455D0A16

!

!

ip dhcp excluded-address 192.168.5.65 192.168.5.70

!

ip dhcp pool LAN1

network 192.168.5.64 255.255.255.224  default-router 192.168.5.94  dns-server 8.8.8.8

!

!

!

no ip cef ipv6 unicast-routing no ipv6 cef

!

!

!

!

license udi pid CISCO1941/K9 sn FTX15249UMV-

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

interface GigabitEthernet0/0  ip address 192.168.5.2 255.255.255.192  duplex auto  speed auto  ipv6 enable  ipv6 dhcp server Stateful  standby version 2  standby 1 preempt  standby 0 ip 192.168.5.94  standby priority 101  standby preempt

!

interface GigabitEthernet0/0.40  encapsulation dot1Q 40  ip address 192.168.5.65 255.255.255.224  ipv6 address 2001:ACAD:DB8:5::2/64  ipv6 enable  ipv6 ospf 1 area 0  standby version 2

interface GigabitEthernet0/1  no ip address  duplex auto  speed auto  shutdown

!

interface Serial0/1/0  ip address 20.0.0.2 255.255.255.252  clock rate 2000000

!

interface Serial0/1/1  no ip address  clock rate 2000000  shutdown

!

interface Vlan1  no ip address  shutdown

!

router ospf 1  router-id 1.1.1.1  log-adjacency-changes  network 20.0.0.0 0.0.0.3 area 0  network 192.168.5.0 0.0.0.63 area 0  network 192.168.5.64 0.0.0.31 area 0

!

ipv6 router ospf 1  router-id 1.1.1.1  log-adjacency-changes

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

!

line con 0

password 7 082243401A160912

login

!

line aux 0

!

line vty 0 4

password 7 0822404F1A0A

login line vty 5 15  password 7 0822404F1A0A

login

!

!

!

end

DeliveryR2:

!

version 15.1

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

!

hostname DeliveryR2

!

!

!

enable password 7 0822455D0A16

!

!

ip dhcp excluded-address 192.168.5.97 192.168.5.102

!

ip dhcp pool LAN2

network 192.168.5.96 255.255.255.240  default-router 192.168.5.94  dns-server 8.8.8.8

!

!

!

no ip cef ipv6 unicast-routing

!

no ipv6 cef

!

!

!

!

license udi pid CISCO1941/K9 sn FTX1524O9WR-

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

ip address 192.168.6.1 255.255.255.240  duplex auto  speed auto

!

interface GigabitEthernet0/1  ip address 192.168.5.3 255.255.255.192  duplex auto  speed auto  standby version 2  standby 0 ip 192.168.5.94  standby preempt

!

interface GigabitEthernet0/1.50  encapsulation dot1Q 50  ip address 192.168.5.97 255.255.255.240  ipv6 address 2001:ACAD:DB8:5::1/64  ipv6 ospf 1 area 0  standby version 2

!

interface Serial0/1/0  no ip address  clock rate 2000000  shutdown

!

interface Serial0/1/1  ip address 30.0.0.2 255.255.255.252  clock rate 2000000

!

interface Vlan1  no ip address  shutdown

!

router ospf 2  router-id 2.2.2.2  log-adjacency-changes  network 30.0.0.0 0.0.0.3 area 0  network 192.168.5.0 0.0.0.63 area 0  network 192.168.5.96 0.0.0.15 area 0  network 192.168.6.0 0.0.0.15 area 0

!

ipv6 router ospf 1  router-id 2.2.2.2  log-adjacency-changes

!

ip classless

ip route 209.165.201.0 255.255.255.0 192.168.6.2

!

ip flow-export version 9

!

!

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

!

line con 0

password 7 082243401A160912

login

!

line aux 0

!

line vty 0 4

password 7 0822404F1A0A

login

line vty 5 15  password 7 0822404F1A0A

login

!

!

!

end

DeliverySW1:

!

version 15.0

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

!

hostname DeliverySW1

!

enable password 7 0822455D0A16

!

!

!

ip ssh version 2

ip domain-name taco.com

!

username admin secret 5 $1$mERr$oixFHbQeZPA/RsivKDJhf.

!

!

!

spanning-tree mode pvst spanning-tree extend system-id

!

interface Port-channel1  switchport mode trunk

!

interface FastEthernet0/1  switchport mode trunk  channel-group 1 mode auto

!

interface FastEthernet0/2  switchport mode trunk  channel-group 1 mode auto

!

interface FastEthernet0/3  switchport access vlan 40  ip dhcp snooping trust  switchport mode access

!

interface FastEthernet0/4

!

interface FastEthernet0/5  switchport access vlan 40  switchport mode access  switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict

switchport port-security mac-address sticky

00D0.BC5B.D8A9  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/6  switchport access vlan 99  shutdown

!

interface FastEthernet0/7

switchport access vlan 99

shutdown

!

interface FastEthernet0/8  switchport access vlan 99  shutdown

!

interface FastEthernet0/9  switchport access vlan 99  shutdown

!

interface FastEthernet0/10  switchport access vlan 40  switchport mode access  switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict   switchport port-security mac-address sticky

0001.646A.BB84  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/11  switchport access vlan 99  shutdown

!

interface FastEthernet0/12  switchport access vlan 99  shutdown

!

interface FastEthernet0/13  switchport access vlan 99  shutdown

!

interface FastEthernet0/14  switchport access vlan 99  shutdown

!

interface FastEthernet0/15  switchport access vlan 99  shutdown

!

interface FastEthernet0/16  switchport access vlan 99 shutdown

!

interface FastEthernet0/17  switchport access vlan 99  shutdown

!

interface FastEthernet0/18  switchport access vlan 99  shutdown

!

interface FastEthernet0/19  switchport access vlan 99  shutdown

!

interface FastEthernet0/20  switchport access vlan 99  shutdown

!

interface FastEthernet0/21  switchport access vlan 99  shutdown

!

interface FastEthernet0/22  switchport access vlan 99  shutdown

!

interface FastEthernet0/23  switchport access vlan 99  shutdown

!

interface FastEthernet0/24  switchport access vlan 99  shutdown

!

interface GigabitEthernet0/1  ip dhcp snooping trust  switchport mode trunk

!

interface GigabitEthernet0/2  switchport access vlan 99  shutdown

!

interface Vlan1  no ip address shutdown

!

interface Vlan40

ip address 192.168.5.73 255.255.255.224

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

line con 0

password 7 082243401A160912

login

!

line vty 0 4

password 7 0822404F1A0A

login local  transport input ssh line vty 5 15  password 7 0822404F1A0A

login local  transport input ssh !

!

!

!

end

DeliverySW2:

!

version 15.0

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

!

hostname DeliverySW2

!

enable password 7 0822455D0A16

!

!

!

ip ssh version 2 ip domain-name taco.com

!

username admin secret 5 $1$mERr$vTbHul1N28cEp8lkLqr0f/

!

!

!

spanning-tree mode pvst spanning-tree extend system-id

!

interface Port-channel1  switchport mode trunk

!

interface FastEthernet0/1  switchport mode trunk  channel-group 1 mode desirable

!

interface FastEthernet0/2  switchport mode trunk  channel-group 1 mode desirable

!

interface FastEthernet0/3  switchport access vlan 99  shutdown

!

interface FastEthernet0/4  switchport access vlan 99  shutdown

!

interface FastEthernet0/5  switchport access vlan 50  switchport mode access  switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict   switchport port-security mac-address sticky

0060.7097.8110  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/6  switchport access vlan 99  shutdown

!

interface FastEthernet0/7  switchport access vlan 99  shutdown

!

interface FastEthernet0/8  switchport access vlan 99  shutdown

!

interface FastEthernet0/9  switchport access vlan 99  shutdown

!

interface FastEthernet0/10  switchport access vlan 50  switchport mode access  switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict

switchport port-security mac-address sticky

00D0.58D2.83A7  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/11

switchport access vlan 99  shutdown

!

interface FastEthernet0/12  switchport access vlan 99  shutdown

!

interface FastEthernet0/13  switchport access vlan 99  shutdown

!

interface FastEthernet0/14  switchport access vlan 99  shutdown

!

interface FastEthernet0/15  switchport access vlan 99  shutdown

!

interface FastEthernet0/16  switchport access vlan 99  shutdown

interface FastEthernet0/17  switchport access vlan 99  shutdown

!

interface FastEthernet0/18  switchport access vlan 99  shutdown

!

interface FastEthernet0/19  switchport access vlan 99  shutdown

!

interface FastEthernet0/20  switchport access vlan 99  shutdown

!

interface FastEthernet0/21  switchport access vlan 99  shutdown

!

interface FastEthernet0/22  switchport access vlan 99  shutdown

!

interface FastEthernet0/23  switchport access vlan 99  shutdown

!

interface FastEthernet0/24  switchport access vlan 99  shutdown

!

interface GigabitEthernet0/1  ip dhcp snooping trust  switchport mode trunk

!

interface GigabitEthernet0/2  switchport access vlan 99  shutdown

!

interface Vlan1  no ip address  shutdown interface Vlan50

ip address 192.168.5.109 255.255.255.240

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

line con 0

password 7 082243401A160912

login

!

line vty 0 4

password 7 0822404F1A0A

login local  transport input ssh line vty 5 15  password 7 0822404F1A0A

login local  transport input ssh

!

!

!

end

ConR:

!

version 15.1

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

!

hostname ConR

!

!

!

enable password 7 0822455D0A16

!

!

!

!

!

ip cef ipv6 unicast-routing

!

no ipv6 cef

!

!

!

!

license udi pid CISCO1941/K9 sn FTX1524P5FJ-

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

interface GigabitEthernet0/0  ip address 192.168.6.2 255.255.255.240  ip nat outside  duplex auto  speed auto

!

interface GigabitEthernet0/0.60  no ip address

!

interface GigabitEthernet0/1  ip address 192.168.6.33 255.255.255.240  ip nat inside  duplex auto  speed auto

!

interface GigabitEthernet0/1.60

encapsulation dot1Q 60

ip address 192.168.6.17 255.255.255.240  ipv6 enable  ipv6 ospf 1 area 0

!

interface Vlan1  no ip address  shutdown

!

router ospf 3  router-id 3.3.3.3  log-adjacency-changes  network 192.168.6.32 0.0.0.15 area 0  network 192.168.6.16 0.0.0.15 area 0  network 192.168.6.0 0.0.0.15 area 0

!

ipv6 router ospf 1  router-id 3.3.3.3  log-adjacency-changes

!

ip nat inside source static 192.168.6.19 209.165.201.5  ip classless

ip route 192.168.5.96 255.255.255.240 192.168.6.1

!

ip flow-export version 9

!

!

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

!

line con 0

password 7 082243401A160912

login

!

line aux 0

!

line vty 0 4

password 7 0822404F1A0A

login line vty 5 15 password 7 0822404F1A0A

login

!

!

!

end

ConSW:

!

version 15.0

no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption

!

hostname ConSW

!

enable password 7 0822455D0A16

!

!

!

ip ssh version 2 ip domain-name taco.com !

username admin secret 5 $1$mERr$vTbHul1N28cEp8lkLqr0f/

!

!

!

spanning-tree mode pvst spanning-tree extend system-id

!

interface FastEthernet0/1  switchport access vlan 60  switchport mode access  switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict   switchport port-security mac-address sticky

0002.1682.947A  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/2  switchport access vlan 60 switchport mode access switchport port-security

switchport port-security mac-address sticky   switchport port-security violation restrict   switchport port-security mac-address sticky

0002.4A95.CB1A  spanning-tree portfast  spanning-tree bpduguard enable

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20 !

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1  switchport mode trunk

!

interface GigabitEthernet0/2

!

interface Vlan1  no ip address  shutdown

!

interface Vlan60

ip address 192.168.6.18 255.255.255.240

!

banner motd !WARNING. You need authorization for this

device!

!

!

!

line con 0

password 7 082243401A160912

login

!

line vty 0 4

password 7 0822404F1A0A

login local  transport input ssh line vty 5 15  password 7 0822404F1A0A

login local  transport input ssh

!

!

!

!

end